# Build a Night Security Light with Arduino

In this project you’re going to build a night security light with a relay module, a photoresistor and an Arduino.

A night security light only turns on when it’s dark and when movement is detected.

Here’s the main features of this project:

- the lamp turns on when it’s dark AND movement is detected;

- when movement is detected the lamp stays on for 10 seconds;

- when the lamp is ON and detects movement, it starts counting 10 seconds again;

- when there’s light, the lamp is turned off, even when motion is detected.

## Recommended resources

The following resources include guides on how to use the relay module and the PIR motion sensor with the Arduino, which might be useful for this project.

- Guide for Relay Module with Arduino

- Arduino with PIR Motion Sensor

## Parts required

Here’s a complete list of the parts required for this project:

- Arduino UNO – read Best Arduino Starter Kits

- PIR Motion Sensor

- Photoresistor

- 10kOhm resistor

- Relay module

- Lamp cord set (view on eBay)

- Breadboard

- Jumper wires

Besides these electronics components, you also need an AC male socket, an AC wire and a lamp bulb holder (a lamp cord set). My lamp cord set is the one in the figure below.

You can use the preceding links or go directly to MakerAdvisor.com/tools to find all the parts for your projects at the best price!

## Code

Download or copy the following code to your Arduino IDE, and upload it to your Arduino board.

Warning: do not upload a new code to your Arduino board while your lamp is connected to the mains voltage. You should unplug the lamp from mains voltage, before upload a new sketch to your Arduino.

/*

* Rui Santos

* Complete Project Details https://randomnerdtutorials.com

*/

// Relay pin is controlled with D8. The active wire is connected to Normally Closed and common

int relay = 8;

volatile byte relayState = LOW;

// PIR Motion Sensor is connected to D2.

int PIRInterrupt = 2;

// LDR pin is connected to Analog 0

int LDRPin = A0;

// LDR value is stored on LDR reading

int LDRReading;

// LDR Threshold value

int LDRThreshold = 300;

// Timer Variables

long lastDebounceTime = 0;

long debounceDelay = 10000;

void setup() {

// Pin for relay module set as output

pinMode(relay, OUTPUT);

digitalWrite(relay, HIGH);

// PIR motion sensor set as an input

pinMode(PIRInterrupt, INPUT);

// Triggers detectMotion function on rising mode to turn the relay on, if the condition is met

attachInterrupt(digitalPinToInterrupt(PIRInterrupt), detectMotion, RISING);

// Serial communication for debugging purposes

Serial.begin(9600);

}

void loop() {

// If 10 seconds have passed, the relay is turned off

if((millis() - lastDebounceTime) > debounceDelay && relayState == HIGH){

digitalWrite(relay, HIGH);

relayState = LOW;

Serial.println("OFF");

}

delay(50);

}

void detectMotion() {

Serial.println("Motion");

LDRReading = analogRead(LDRPin);

// LDR Reading value is printed on serial monitor, useful to get your LDRThreshold

//Serial.println(LDRReading);

// Only turns the Relay on if the LDR reading is higher than the LDRThreshold

if(LDRReading > LDRThreshold){

if(relayState == LOW){

digitalWrite(relay, LOW);

}

relayState = HIGH;

Serial.println("ON");

lastDebounceTime = millis();

}

}

View raw code

## Schematics

Here’s the schematics for this project.

Note: if you have an earth (GND) connection in the mains voltage cable – a yellow and green cable – it should go outside the relay module, like the blue wire (neutral).

## Demonstration

Here’s your circuit in action:

## Wrapping up

In this project you’ve built a night security light with a photoresistor and a PIR motion sensor.

This is a great project to practice with relays and with the PIR motion sensor.

If you like Arduino projects, make sure you check our latest Arduino course: Arduino Step-by-step Projects – Build 25 Projects

Thanks for reading,

Sara Santos